HAMLET FIRE DOOR GLASS FITTING:

In the event of the door glass being broken it can easily be replaced:

- The door(s) should be lifted off the hinges so that the above operations can be carried out on a workbench or similar level surface. This should be done when the appliance is cold (Fig.1)

Fig. 1. Removal of Hamlet Hero door

- Unscrew the two nuts, on the inside of the door securing the retaining clips.

- Carefully remove any pieces of broken glass, and sealing gasket wearing suitable gloves.

Fig. 2. Lining up the gasket

- Re-place the gaskets and line them up with in relation to the door frame. (Fig.2)

Fig. 3. Fitting new door gasket

- Re-seat the new glass, ensuring the sealing gasket is flat and in contact with the glass. (Fig 3)

Fig. 4. Fitting new glass

- Replace the two retaining clips and nuts. Do not over-tighten the nuts as damage may occur to the glass (Fig.4)

- Place the door carefully in line with the hinges, ensuring that both hinges sit square to the body; if not repeat steps.